
	Pos.:
	Description
	Qty
	ME
	Unit price


	Total price



	
	
	

	
	Salamander Salvis Classic Pro with HiLight heating element

Specification:


planed:

Capacity:
:

1 x GN 1/1 or 2 plates Ø 32

Dimensions (L/D/H):

600 x 480/520 x 445 mm

Weight net/gross:
:

41 / 50 kg

Connected load:


4.6 kW

Voltage:



400 V  2N AC

Fuse protection:


2 x 16 A

Security approvals:

EN 60335-2-48

EMV-tested:


EN 55014-1

Protection:


IPX3


	offered:

...........................................

...........................................

...........................................

...........................................

...........................................

...........................................

...........................................

………………………………
...........................................



	                    Salamander with HiLight heating element

	
	Special Features
· Salamander to gratinate, scallop and keep foods warm in the à-la-carte business.
· Can be equipped from three sides and is height adjustable; hence, the food is always visible.
· The heater reflector with its two separately controllable HiLight heater systems is steplessly adjustable between 73 and 166 mm in its height, which makes it very adaptable to the respective types of food.
· HiLight heaters offer short heating times, operating readiness and the best browning results.
· Operated via a selection switch and membrane keys for Start/Stop and Timer keys with digital display.
· Manual or automatic operation with electronic timer from 0 to 20 minutes; automatic shutoff with acoustic signal after time has elapsed, timer memory function and plate detection.
· This function with automatic plate detection is an advantage regarding operator comfort and energy conservation.
· Select from 4 heating levels (full/left/right/keep warm) - can be switched individually and for energy saving.
· Capacity for GN 1/1 or 2 plates with diameters of 320 mm.
Equipment/Model:
Control:
· Operating mode control: Full performance, right, left, keep warm
· 2 indicator lamps
Housing:
· Model is completely made of stainless steel
· Prepared to be set up on a table (standard) or on a wall or ceiling
· 2 Hi-Light heaters with 2.3 kW heating power each
· Height adjustable heater reflector with double coating
· 3 sides, circumferential, air-cooled handle on heater reflector
· Integrated GN sheet metal in floor part
· Transport latch
Safety:
· CE conformity as per EN 60334-2-48
· General product safety as per EC directives 92/59/EWG
· Low voltage directive 89/336/EWG
· Food industry hygiene EC directive 93/43
· SEV test mark
Documentation:
· Installation and Operating Manual
Optional equipment and system accessories:
· Installation kits for wall and ceiling installation
· Temperature protection sheet for wall installation
Technical amendments reserved!

	
	

	
	Manufacturer: Salvis AG, Oftringen

	_____
	____
	_________
	_________


[image: image1.jpg]smart cooking



Version 02/2025
1/2

